SLAM Map Evaluation

2D Occupancy Grid · 3D Point Cloud

Walle GCS  ·  v1.0  ·  2026-06-05

# 1. 2D SLAM — Occupancy Map

| Metric | How | Target |
| --- | --- | --- |
| Scale error | landmark distance on grid vs. surveyed | <= 2% |
| Wall straightness | RMS residual of fit to a straight wall | <= 1 cell |
| Loop-closure gap | start->end pose drift / path length | <= 2% |
| Yaw drift | heading error after a closed loop | <= 5 deg |
| Map completeness | swept area marked known | >= 90% |
| Free-space precision | cells marked free that truly are | >= 95% (safety) |
| Obstacle recall | real obstacles marked occupied | >= 90% (safety) |
| VO inlier ratio | Inliers / Matches from stats | >= 0.5 sustained |

# 2. 3D Reconstruction — Point Cloud

| Metric | How | Target |
| --- | --- | --- |
| Cloud-to-ref RMS | nearest-neighbour distance after ICP | <= 3 cm indoor |
| Scale error | known object dimension in cloud vs. truth | <= 2% |
| Planarity RMS | fit plane to flat surface, residual | <= 2 cm |
| Completeness | reference surface within tau of a point | >= 85% |
| Outlier rate | floaters > 5x median NN distance | <= 2% |
| Point count / conf | N points above export confidence cutoff | report |

# 3. Cross-Cutting & Verdict

- Performance: end-to-end FPS, frame-drop rate (< 10%), CPU/GPU budget, no thermal throttle.

- Determinism: run each sequence x3; key-metric spread < 1%.

- Robustness: recovery from hard turns, occlusion, low light - logged.

# 4. Test Protocol

- Freeze a test set per scene class: 1 loop, 1 out-and-back, 1 long straight run (indoor + outdoor).

- Record sensor, HFOV, scene/variant, lighting, motion, dynamic objects for each run.

- Capture ground truth: surveyed distances, reference LiDAR/photogrammetry scan, or loop self-consistency.

- Run the pipeline; log the stats dict to CSV per frame; save .npy grid and .ply.

- Score with the eval notebooks / scorer; fill the scorecard; attach artifacts.

- Track metrics build-over-build for regression trend lines.