Walle GCS

Perception & SLAM Evaluation Plan

Depth Perception · Object Detection · 2D SLAM · 3D Reconstruction

Document version 1.0  ·  2026-06-05
Ground Control Station — Drone Autonomy Stack

# 1. What We Are Trying To Do

Walle GCS is the ground control station for an autonomous drone. From a single monocular video feed it must build a usable map of the environment and understand the people and objects in it, in real time, without stalling the operator GUI. Four perception subsystems run alongside the video loop:

| Subsystem | What it produces | Why it matters |
| --- | --- | --- |
| Depth perception | Per-pixel metric depth (DA V2) | Feeds SLAM unprojection and obstacle ranging |
| 2D SLAM | Log-odds occupancy grid + pose track | Live navigation / free-space map |
| 3D reconstruction | Fused .ply point cloud | Survey / inspection deliverable |
| Object detection | Human tracks + identities | Situational awareness, follow/track behaviours |

This document defines the evaluation goal for each subsystem: turning “the map looks about right” into repeatable, numeric pass/fail criteria, and recording which models we use and how we benchmark them.

# 2. Models Under Evaluation

## 2.1 Depth Perception

The SLAM front-end uses Depth Anything V2 (metric variants) selected by scene and size. The full comparison study (Eval_models/depth_models_comparison.ipynb) also benchmarked legacy baselines.

| Role | Model | Notes |
| --- | --- | --- |
| Production (indoor) | Depth-Anything-V2-Metric-Indoor-Small / Base (hf) | fp16 GPU, input 322px, used in slam_engine.py |
| Production (outdoor) | Depth-Anything-V2-Metric-Outdoor-Small / Base (hf) | scene-switched via DEPTH_MODELS_SLAM |
| Baseline (compared) | Depth Anything V1, DPT-Hybrid, MiDaS | reference points in the comparison notebook |

## 2.2 Object Detection & Identity

| Role | Model / Method | Source |
| --- | --- | --- |
| Human detection + pose | YOLOv8-pose (yolov8n-pose.pt) | human_tracker_engine.py |
| Multi-object tracking | DeepSort (deep_sort_realtime) | ID-stable tracks across frames |
| Trajectory prediction | Kalman + motion classifier | predictive follow behaviour |
| Face detection | SCRFD-10g (InsightFace buffalo_l, ONNX) | face_recognition_engine.py |
| Face recognition | ResNet50 ArcFace (buffalo_l, ONNX) | cosine match vs. enrolled embeddings |

## 2.3 3D Reconstruction

| Role | Model | Source |
| --- | --- | --- |
| Streaming 3D map | LingBot-Map (lingbot-map-long.pt) | reconstruction_engine.py → fused .ply |

# 3. How We Evaluate — Depth Perception

Depth is scored against ground-truth depth (RGB-D capture or LiDAR) using the standard monocular-depth metrics computed in Eval_models/eval.ipynb. Because the models are metric, we evaluate without per-image median rescaling.

| Metric | Meaning | Target (indoor) |
| --- | --- | --- |
| AbsRel | mean ¦d_pred − d_gt¦ / d_gt | ≤ 0.10 |
| RMSE (m) | root-mean-square depth error | ≤ 0.40 |
| δ₁ (delta_1) | % pixels with max(d/d*, d*/d) < 1.25 | ≥ 0.90 |
| δ₂ (d2) | same, threshold 1.25² | ≥ 0.97 |
| Edge accuracy | depth sharpness at object boundaries | qualitative, no smearing |
| Latency / FPS | fp16 GPU inference rate at 322px | ≥ real-time target |

Selection rule: pick the smallest model whose AbsRel / δ₁ clear target at the required FPS. Small wins unless Base measurably improves SLAM map quality (§5).

# 4. How We Evaluate — Object Detection & Identity

## 4.1 Detection (YOLOv8-pose)

| Metric | Meaning | Target |
| --- | --- | --- |
| mAP@0.5 | detection accuracy vs. labelled persons | ≥ 0.85 (person class) |
| Precision / Recall | at conf 0.35 (engine default) | P ≥ 0.90, R ≥ 0.85 |
| Keypoint OKS | pose-keypoint accuracy | ≥ 0.80 |
| Inference FPS | GPU throughput, cadence-capped | ≥ real-time target |

## 4.2 Tracking (DeepSort)

| Metric | Meaning | Target |
| --- | --- | --- |
| MOTA | multi-object tracking accuracy | ≥ 0.70 |
| ID switches | identity swaps per track over a run | minimise; report count |
| Track continuity | % frames a true person holds one ID | ≥ 90% |

## 4.3 Face Recognition (SCRFD + ArcFace)

| Metric | Meaning | Target |
| --- | --- | --- |
| Detection recall | faces found vs. present (multi-scale) | ≥ 0.95 |
| Verification TAR@FAR | true-accept at fixed false-accept rate | TAR ≥ 0.95 @ FAR 1e-3 |
| Rank-1 ID accuracy | correct identity vs. enrolled gallery | ≥ 0.95 |
| Cosine threshold | operating point on match score | tuned on held-out set |

# 5. How We Evaluate — 2D SLAM (Occupancy Map)

The 2D map is a GPU log-odds occupancy grid (map_<ts>_logodds.npy). Metrics load the raw grid (sigmoid → probability; occupied > 0.60, free < 0.40, matching the engine) and use the per-frame pose telemetry from the stats dict.

| Metric | How | Target |
| --- | --- | --- |
| Scale error | landmark distance on grid vs. surveyed | ≤ 2% |
| Wall straightness | RMS residual of fit to a straight wall | ≤ 1 cell |
| Loop-closure gap | start→end pose drift / path length | ≤ 2% |
| Yaw drift | heading error after a closed loop | ≤ 5° |
| Map completeness | swept area marked known | ≥ 90% |
| Free-space precision | cells marked free that truly are | ≥ 95% (safety) |
| Obstacle recall | real obstacles marked occupied | ≥ 90% (safety) |
| VO inlier ratio | Inliers / Matches from stats | ≥ 0.5 sustained |

# 6. How We Evaluate — 3D Reconstruction (Point Cloud)

The fused .ply is registered to a reference scan (ICP) and scored on geometric fidelity, coverage, and cleanliness.

| Metric | How | Target |
| --- | --- | --- |
| Cloud-to-ref RMS | nearest-neighbour distance after ICP | ≤ 3 cm indoor |
| Scale error | known object dimension in cloud vs. truth | ≤ 2% |
| Planarity RMS | fit plane to flat surface, residual | ≤ 2 cm |
| Completeness | reference surface within τ of a point | ≥ 85% |
| Outlier rate | floaters > 5× median NN distance | ≤ 2% |
| Point count / conf | N points above export confidence cutoff | report |

# 7. Cross-Cutting & Verdict

Performance: end-to-end FPS, frame-drop rate (< 10%), CPU/GPU budget, no thermal throttle.

Determinism: run each sequence ×3; key-metric spread < 1%.

Robustness: recovery from hard turns, occlusion, low light — logged.

Verdict rule:

PASS — all geometric-accuracy rows green.

FAIL — any single safety row red (free-space precision, obstacle recall, cloud-to-ref).

CONDITIONAL — outputs green but process rows (inliers, FPS) red: works but fragile.

# 8. Test Protocol

Freeze a test set per scene class: 1 loop, 1 out-and-back, 1 long straight run (indoor + outdoor).

Record sensor, HFOV, scene/variant, lighting, motion, dynamic objects for each run.

Capture ground truth: surveyed distances, reference LiDAR/photogrammetry scan, or loop self-consistency.

Run the pipeline; log the stats dict to CSV per frame; save .npy grid and .ply.

Score with the eval notebooks / scorer; fill the scorecard; attach artifacts.

Track metrics build-over-build for regression trend lines.

Companion: docs/slam_evaluation.md holds the full per-run scorecard template and the proposed eval_slam.py scorer.